DA40-180 CHECKLIST

REVISED ON 5.8.2026

# PREFLIGHT

DOCUMENTS......................................VERIFIED

W/B...................................................COMPLETE

T/O DISTANCE.................................COMPLETE

LANDING DISTANCE.......................COMPLETE

Va.....................................................COMPUTED

# INTERIOR

CONTROL LOCK...............................REMOVED

FLIGHT CONTROLS...............................CHECK

MAGS............................................................OFF

CANOPY & WINDOWS............................CLEAN

SWITCHES...................................................OFF

BREAKERS.........................................CHECK IN

CONTROL LEVERS...................................FREE

THROTTLE..................................................IDLE

PROPELLER................................................FWD

MIXTURE...............................................CUTOFF

BATTERY MASTER........................................ON

INTERIOR LIGHTS..................................CHECK

EXTERIOR LIGHTS................................CHECK

PITOT HEAT............................................CHECK

TRIM....................................................NEUTRAL

FLAPS...........................................................LDG

FUEL QUANTITY.....................................CHECK

TACH....................................................RECORD

BATTERY MASTER......................................OFF

FIRE EXTINGUISHER.............................CHECK

BAGGAGE............................................STOWED

# EXTERIOR

LEFT WING

PILOT STEP.............................................CHECK

AIR INLET................................................CHECK

STRUT.....................................................CHECK

TIRE PRESS..................36 PSI..................CHECK
TIRE/WHEEL...........................................CHECK

BRAKE.....................................................CHECK

CHOCKS...............................................REMOVE

FUEL DRAIN..............................................SUMP

STALL WARNING....................................CHECK

FUEL QUANTITY.....................................CHECK

FUEL CAP..............................................SECURE

FUEL VENT..............................................CHECK

PITOT PROBE.........................................CHECK

LEFT WING..............................................CHECK

STALL STRIPS........................................CHECK

TAXI/LANDING LIGHT.............................CHECK

POS/STROBE..........................................CHECK

WING TIP.................................................CHECK

TIEDOWN.............................................REMOVE

AILERON.................................................CHECK

FLAP.........................................................CHECK

LEFT FUSELAGE

REAR CANOPY........................................CHECK

SKIN..........................................................CHECK

A/P STATIC...............................................CHECK

ANTENNAS..............................................CHECK

EMPENNAGE

RUDDER...................................................CHECK

RUDDER TRIM TAB.................................CHECK

STABILIZER..............................................CHECKSTABILIZER TRIM TAB............................CHECK

TAIL SKID/LOWER FIN............................CHECK

TIEDOWN..............................................REMOVE

RIGHT FUSELAGE

WINDOW/CANOPY..................................CHECK

SKIN..........................................................CHECK

A/P STATIC...............................................CHECK

RIGHT WING

FLAP.........................................................CHECK

AILERON..................................................CHECK

TIEDOWN..............................................REMOVEWING TIP..................................................CHECK

POSITION/STROBE.................................CHECK

STALL STRIPS.........................................CHECK

RIGHT WING............................................CHECK

FUEL VENT...............................................CHECK

FUEL QUANTITY......................................CHECK

FUEL CAP..............................................SECURE

FUEL DRAIN...............................................SUMP

STRUT......................................................CHECK

TIRE PRESS..................36 PSI..................CHECK
TIRE/WHEEL...........................................CHECK

BRAKE.....................................................CHECK

CHOCKS...............................................REMOVE

CO-PILOT STEP......................................CHECK

NOSE AREA

OIL...........................MIN 6 / MAX 8...............CHECK

COWLING.................................................CHECK

AIR INLETS...............................................CLEAR

PROPELLER............................................CHECK

SPINNER..................................................CHECK

TIRE PRESS...................29 PSI.................CHECK

TIRE/WHEEL..............…………………….CHECK

NOSE GEAR.............................................CHECK

FUEL DRAIN...................……..…………….SUMP

WINDSCREEN..........................................CLEAN

# MANEUVER FLOWS

TAKEOFF

TRIM…..........................................................SET

PROPELLER...................….........................FWD

MIXTURE...................................................…SET

FLAPS........................................................…T/O

FUEL PUMP....................................................ON

CRUISE

TRIM..............................................................SET

PROP.............................................................SET

MIXTURE......................................................SET

FLAPS....................................................CRUISE

FUEL PUMP....................................................ON

LANDING

TRIM..............................................................SET

PROP............................................................FWD

MIXTURE...........................................ENRICHEN

FLAPS.........................................T/O < 108 KIAS

.....................................................LDG < 91 KIAS

# INSTRUMENT FLOWS

BEFORE IAF

WX→ATIS/AWOS...................................OBTAIN
ALTIMETER..................................................SET

RADIOS.........................................................SET

NAVS.............................................................SET

BRIEF...............................................COMPLETE

MIXTURE...........................................ENRICHEN

THROTTLE.............................................AS REQ

LIGHTS...................................................AS REQ

BEFORE FAF

CLEARANCE..........................................OBTAIN

FUEL PUMP....................................................ON

FUEL SELECTION.....................FULLEST TANK

MIXTURE......................................................SET

PROP......................................................AS REQ

FLAPS……………...0.5NM – 1NM….....………..T/O

500’ ABOVE MINS

PROP............................................................FWD

AIRSPEED..............................................67 KIAS

FLAPS...........................................................LDG

LANDING LIGHT.............................................ON

MINIMUMS – CONTINUING

GUMPS....................................................CHECK

AIRSPEED..............................................67 KIAS

FLAPS...........................................................LDG

LANDING LIGHT.............................................ON

MINIMUMS – MISSED

THROTTLE..................................................FULL

FLAPS............................................................T/O

AIRSPEED..............................................66 KIAS

REPORT..................................................CHECK

RADIOS.........................................................SET

# PASSENGER BRIEF

Seat Belts/Position...............................CHECK

Air Vents..............................................CHECK

Fire Extinguisher..................................CHECK

Exits/Emergency Plan..........................CHECK

Talking/Traffic Scanning......................CHECK

Your Questions....................................CHECK

# C–GUMPS

CLEARANCE......................................OBTAIN

GAS.....................................................CHECK

UNDERCARRIAGE..........................WELDED

MIXTURE..................................................SET

POWER..............................................AS REQ

PROP.............................................FULL FWD

SWITCHES/SEATBELTS....................CHECK

# V SPEEDS

VSO.....................................................52 KIAS

VS.......................................................53 KIAS

VX.......................................................66 KIAS

VY.......................................................73 KIAS

VG.......................................................73 KIAS

VFE – LDG..........................................91 KIAS

VFE – T/O..........................................108 KIAS

VA................1720 LBS - 2284 LBS.................94 KIAS

VA................2284 LBS - 2646 LBS...............111 KIAS

VNO....................................................129 KIAS

VNE....................................................178 KIAS

# PATTERN SPEEDS

DOWNWIND.......................................95 KIAS

BASE..................................................85 KIAS

FINAL..................................................75 KIAS

SHORT FINAL....................................67 KIAS

# LOCAL FREQUENCIES

ATIS....................................................118.55 Ground..................................................121.7 Tower.....................................................118.1 Practice Area......................................122.75 Flight Service........................................122.6 Center...................................................134.5 Approach/Departure..............................119.7

# OFFICE #               970.254.0444

IF YOU ARE UNABLE TO RETURN TO GRAND JUNCTION WHEN EXPECTED/A LANDING IS REQUIRED AT ANOTHER AIRPORT DUE TO MECHANICAL ISSUES OR WEATHER, PLEASE CLOSE YOUR FLIGHT PLAN, THEN CALL YOUR INSTRUCTOR OR CHIEF FLIGHT INSTRUCTOR AS SOON AS PRACTICAL.

# BEFORE ENGINE START

PREFLIGHT INSPECTION.............COMPLETE

PEDALS.....................................ADJUST/LOCK

PASSENGER BRIEF.......................COMPLETE

SEAT BELTS....................................FASTENED

BAGGAGE...........................................SECURE

REAR CANOPY...................................SECURE

FRONT CANOPY.................................SECURE

PARKING BRAKE........................................SET

FLIGHT CONTROLS.........................CORRECT

TRIM.............................................................T/O

THROTTLE.................................................IDLE

PROPELLER...............................................FWD

MIXTURE...................................................LEAN

FRICTION.....................................................ADJ

ALT AIR................................................CLOSED

ALT STATIC..........................................CLOSED

AVIONICS MASTER....................................OFF

ESS BUS......................................................OFF

BATTERY MASTER.......................................ON

ANNUNCIATORS...................................CHECK

FUEL SELECTOR............................FULL TANK

# ENGINE START

THROTTLE.................................................IDLE

BRAKES....................................................HOLD

PROP AREA...........................................CLEAR

STROBES......................................................ON

FUEL PUMP...................................................ON

COLD

THROTTLE........................................1.5” OPEN

MIXTURE...............RICH 3-5 SEC...............LEAN

THROTTLE........................................0.5” OPEN

MAGS......................................................START

HOT

THROTTLE...........................................1” OPEN

MIXTURE......................................................LEAN

MAGS.......................................................START

FLOODED

MIXTURE..................................................LEAN

THROTTLE..................................................MID

MAGS......................................................START

WHEN ENGINE FIRES

MAGS........................................................BOTH

MIXTURE.............MOVE RAPIDLY..............RICH

THROTTLE........................................1000 RPM

DO NOT EXCEED 1500RPM
UNTIL OIL > 100°

OIL PRESSURE............................GREEN ARC

FUEL PUMP.................................................OFF

ALT MASTER................................................ON

AMMETER..............................................CHECK

FUEL PRES.............14 PSI – 35 PSI............CHECK

ANNUNCIATOR......................................CHECK

MIXTURE...............................................AS REQ

# BEFORE TAXI

AVIONICS MASTER......................................ON

WX→ATIS/AWOS..................................OBTAIN

ALTIMETER..................................................SET

GPS/RAIM........................................AVAILABLE

LRU/ANNUCIATORS..............................CHECK

A/P TEST.............................................ENGAGE

...............................................HDG & VS MODE

............................................OVERRIDE CHECK

.....................................DISCONNECT SWITCH

ELECTRIC TRIM....................................CHECK

FLAPS............................................................UP

OIL TEMP.............................................MIN 100°

TAXI LIGHT...................................................ON

PARKING BRAKE...............................RELEASE

CLEARANCE.........................................OBTAIN

BRAKES.................................................CHECK

THROTTLE............................................AS REQ

# INSTRUMENT CHECK

ASI + STBY.............................................CHECK

ATTITUDE + STBY..................................CHECK

ALT + STBY.............................................CHECK

VSI…………………………………………CHECK

HDG → Compass......………………………..SET

OAT.........................................................CHECK

XPDR.....................ALT MODE......................SET

# RUN UP

PARKING BRAKE........................................SET

SEAT BELTS....................................FASTENED

REAR CANOPY...................................SECURE

FRONT CANOPY.................................SECURE

FUEL PUMP...................................................ON

FUEL SELECTOR........................SWAP TANKS

CONTROLS.........................FREE + CORRECT

MIXTURE..........................................FULL RICH

THROTTLE........................................2000 RPM

MIXTURE...................................LEAN FOR ALT

MAGNETOS...............175 MAX...............CHECK

PROPELLER...........REPEAT 3X...............CYCLE

NEVER EXCEED 500 RPM DROP DURING PROP CYCLE

THROTTLE....................................CHECK IDLE

THROTTLE........................................1000 RPM

PARKING BRAKE...............................RELEASE

# PRE-TAKEOFF

TRIM..............................................................T/O

FLAPS...........................................................T/OPROP.................................................FULL FWD

MIXTURE.....................................................SET

FUEL PUMP...................................................ON

FUEL SELECTOR............................FULL TANK

RADIOS........................................................SET

NAV LIGHTS..........................................AS REQ

LANDING LIGHT............................................ON

DEPARTURE BRIEF........................COMPLETE

TIMER..........................................................SET

# NORMAL TAKEOFF

THROTTLE.................................................FULL

ENGINE GAUGES.................................GREEN

AIRSPEED................................................ALIVE

ELEVATOR.........................................NEUTRAL

RUDDER......................................CENTERLINE

ROTATE.................................................59 KIAS

AIRSPEED.....................………………..75 KIAS

400’ AGL

FLAPS.........................…...............................UP

PROP.....................................................AS REQ

# SHORT-FIELD TAKEOFF

BRAKES....................................................HOLD

THROTTLE.................................................FULL

ENGINE GAUGES.................................GREEN

BRAKES.............................................RELEASE

AIRSPEED................................................ALIVE

ELEVATOR.........................................NEUTRAL

RUDDER......................................CENTERLINE

ROTATE.................................................59 KIAS

AIRSPEED.....................………………..66 KIAS

CLEAR OF 50’ OBSTACLE

AIRSPEED.............................................75 KIAS

400’ AGL

FLAPS.........................…...............................UP

PROP.....................................................AS REQ

# CLIMB

# ABOVE 1000’AGL

AIRSPEED.............................................75 KIAS

THROTTLE.................................................FULL

FUEL PUMP...............CHECK DA....................OFF

PROP..................................................2400 RPM

TRIM......................................................AS REQ

ENGINE GAUGES..................................CHECK

LANDING/TAXI LIGHT.................................OFF

# CRUISE

THROTTLE............................................AS REQ

ENGINE GAUGES..................................CHECK

FUEL PUMP...................................................ON

FUEL TANK...............SWAP...............FULL TANK

FUEL PUMP...............CHECK DA....................OFF

MIXTURE.....................................................SET

LEAN PER CRUISE CHART FOR PROPER FUEL FLOW

TRIM...........................................AS REQUIRED

# DESCENT

WX→ATIS/AWOS.................................OBTAIN
ALTIMETER.................................................SET

RADIOS.......................................................SET

NAVS...........................................................SET

BRIEF.............................................COMPLETE

MIXTURE........................................ENRICHEN

THROTTLE...........................................AS REQ

LIGHTS.................................................AS REQ

# APPROACH

CLEARANCE........................................OBTAIN

FUEL PUMP..................................................ON

FUEL SELECTION........SWAP........FULL TANK

MIXTURE....................................................SET

PROP....................................................AS REQ

FLAPS..........................................................T/O

# LANDING

PROP................................................FULL FWD

AIRSPEED............................................67 KIAS

LDG LIGHT...................................................ON

FLAPS.........................................................LDG

# GO-AROUND/MISSED

THROTTLE................................................FULL

FLAPS..........................................................T/O

AIRSPEED............................................66 KIAS

400’ AGL

PROP.....................................................AS REQ

AIRSPEED.............................................75 KIAS

FLAPS.........................…...............................UP

FUEL PUMP...............CHECK DA....................OFF

# AFTER LANDING

BRAKES...............................................AS REQ

FUEL PUMP..............CHECK DA....................OFF

XPDR..........................................................VFR

PITOT HEAT................................................OFF

RADIOS…..….……………………………….SET

FLAPS...........................................................UP

LANDING LIGHT.........................................OFF

TAXI LIGHT...................................................ON

# ENGINE SHUTDOWN

PARKING BRAKE.......................................SET

ENGINE GAUGES.................................CHECK

AVIONICS MASTER...................................OFF

THROTTLE........................................1000RPM

MAGS..........OFF→RPM DROP→BOTH..........CHECK

MIXTURE.............................................CUTOFF

MAGS..........................................................OFF

NAVS/TAXI LIGHT.......................................OFF

TACH TIME.........................................RECORD

ALT/BAT MASTER......................................OFF

CHOCKS.....................................................SET

TIEDOWN.............................................AS REQ

CONTROL LOCK................................SECURE

SECURE AIRCRAFT......................COMPLETE